Electron-Ion Collider Plan

EIC Detector R&D Plan

May 6, 2025

Prepared by:

__________________________________________		Date:__________________

Thomas Ullrich, Title

Reviewed by:

__________________________________________		Date:__________________

Rolf Ent, Experimental Nuclear Physics

__________________________________________		Date:__________________

Elke Aschenauer, Detector Systems

__________________________________________		Date:__________________

Katherine Wilson, Interim Deputy Project Director

__________________________________________		Date:__________________

Luisella Lari, EIC Project Manager

Approved by:

__________________________________________		Date:__________________

Sergei Nagaitsev, EIC Technical Director

This page intentionally left blank, to accommodate back-to-back printing.

REVISION HISTORY

| Revision # | Effective Date | List of Reviewers | Summary of Change |
| --- | --- | --- | --- |
| 00 |  |  | Initial release |

Table of Contents

1.	INTRODUCTION	9

1.1.	Purpose and Scope	9

1.2.	Introduction	9

2.	R&D for Reference Detector	10

2.1.	Tracking	10

2.2.	Particle Identification	14

2.3.	Electromagnetic Calorimetry	20

2.4.	Hadron Calorimetry	22

2.5.	Auxiliary Detectors	23

2.6.	Electronics	25

3.	Generic R&D for Alternative Technologies	26

3.1.	Development of MAPS/DMAPS Sensor for the EIC	26

3.2.	Central Tracker Cylindrical µRWELL	26

3.3.	Central Tracker Cylindrical Micromegas	28

3.4.	GEM-TRD/Tracker	29

3.5.	Time-Of-Flight Detector (ToF)	30

3.6.	Photosensors	31

4.	Generic R&D for Outyears	32

4.1.	MPGD for RICH/Nano-Diamond Materials	32

4.2.	CSGlass for Hadronic Calorimetry	34

4.3.	Particle ID in Barrel for High-pT	34

4.4.	Nanowires	35

4.5.	Radiation Hard  SiPM	35

5.	Appendix	38

6.	Update April 2025	40

List of Acronyms

| AC | Alternating Current |
| --- | --- |
| ADC | Analog Digital Converter |
| ALCOR | A Low-power Chip for Optical sensor Readout (mixed-signal ASIC) |
| ALICE | A Large Ion Collider Experiment, Experiment at LHC, CERN |
| ALPIDE | Monolithic Active Pixel Sensor (MAP) Designed for the ALICE Inner Tracker |
| ANSY | Company Producing Engineering Simulation Software |
| ASIC | Application-Specific Integrated Circuit |
| ASICs | Plural of ASIC |
| ATLAS | A Toroidal LHC Apparatus, Experiment at LHC, CERN |
| BLT | Barrel Timing Later |
| BNL | Brookhaven National Laboratory |
| CALOROC | Calorimeter Readout Chip (ASIC) |
| CD | Critical Decision |
| CEA | Commissariat à l'énergie atomique et aux énergies alternatives, French Alternative Energies and Atomic Energy Commission |
| CERN | European Organization for Nuclear Research |
| CMOS | Complementary Metal–Oxide–Semiconductor |
| CMS | Compact Muon Spectrometer (LHC experiment) |
| COTS | Commercial Off-the-Shelf |
| CRT | Cosmic Ray Telescope |
| CsI | Caesium Iodide |
| CUA | Catholic University of America |
| DAC | Detector Advisory Committee |
| DAQ | Data Acquisition |
| DC | Direct Current |
| DESY | Deutsches Elektronen-Synchrotron |
| DIRC | Detection of Internally Reflected Cherenkov Light |
| DIS | Deep Inelastic Scattering |
| DMAPS | Depleted Monolithic Active Pixel Sensors |
| DOE | U.S. Department of Energy |
| dRICH | Dual-Radiator Ring Imaging Cherenkov Detector |
| DUNE | Deep Underground Neutrino Experiment |
| ECAL | Electromagnetic Calorimeter |
| EIC | Electron-Ion Collider |
| EICROC | EIC Read Out Chip |
| EM | Electromagnetic |
| EMCAL | Electromagnetic Calorimeter |
| ePIC | electron-Proton/Ion Collider |
| ER | Engineering Run |
| FBK | Fondazione Bruno Kessler, Multidisciplinary Research Institution in Italy |
| FCFD | Fermilab Constant Fraction Discriminator ASIC |
| FE | Front-End |
| FEB | Front-End Board |
| FEE | Front-End Electronics |
| FEP | Front-End Processor |
| FIT | Florida Institut of Technology |
| FNAL | Fermi National Accelerator Laboratory |
| FOCAL | Forward Electromagnetic And Hadronic Calorimeter, ALICE Experiment |
| FPGA | Field-Programmable Gate Array |
| FTBF | Fermilab Test Beam Facility |
| GEM | Gas Electron Multiplier, Detector Type |
| GEMS | Gas Electron Multipliers |
| GSI | Helmholtzzentrum für Schwerionenforschung, German Research Institute |
| HCAL | Hadron Calorimeter |
| HEP | High Energy Physics |
| HGCROC | High Granularity Calorimeter Readout Chip (ASIC) |
| HND | Hydrogenated Nano Diamond |
| HPK | Hamamatsu Hotonikusu Kabushiki-Kaisha, Japanese Company also known as Hamamatsu |
| HRPPD | High Rate Picosecond Pixel Detector |
| HV | High Voltage |
| IBF | Ion Back Flow |
| ID | Identification |
| INCOM | Glass and Polymer Microstructures Producing Company in Massachusetts |
| INFN | National Institute for Nuclear Physics, Italian research agency |
| IP | Interaction Point |
| ITS | Inner Tracking System |
| JLAB | Jefferson Laboratory |
| LAPPD | Large Area Picosecond Pixel Detector |
| LAS | Large Area Sensor |
| LBNL | Lawrence Berkeley National Laboratory |
| LFHCAL | Longitudinally Segmented Forward Hadronic Calorimeter (EIC subsystem) |
| LGAD | Low Gain Avalanche Detector |
| LGADS | Plural of LGAD |
| LHC | Large Hadron Collider |
| LpGBT | Low power Gigabit Transceiver |
| LSD | Long Shadow Detector |
| LYSOL | Lu 1.8Y.2SiO5:Ce, Cerium doped Lutetium-Based Scintillation Crystal |
| MAPS | Monolithic Active Pixel Sensors |
| MCP | Multi-Channel Plate |
| MIT | Massachusetts Institute of Technology |
| MLR | Multi-Layer Reticle |
| MM | MicroMegas detector (Micro-Mesh Gaseous Structure) |
| MMG | MicroMegas detector (Micro-Mesh Gaseous Structure) |
| MOSAIX | Stitched MAPS Sensor for ALICE and EIC (under development) |
| MPGD | Micro-Pattern Gas Detector |
| MPW | Multi-Project Wafer |
| mRICH | Modular RICH |
| MTD | MIP (minimum ionizing particle) Timing Detector |
| MQPC | Multi-party Quantum Private Comparison |
| NP | Multi-party Quantum Private Comparison |
| ODU | Old Dominion University |
| ORNL | Oak Ridge National Laboratory |
| PANDA | antiProton ANnihilation at Darmstadt, Experiment at GSI |
| PCB | Printed circuit board |
| PED | Project Engineering and Design |
| PID | Particle Identification |
| PMT | Photomultiplier tube |
| QE | Quantum Efficiency |
| RDO | Readout Board |
| RICH | Ring Imaging Cherenkov |
| SALSA | ASIC readout chip for micro-pattern gas detector |
| SAMPA |  |
| SBS | Super Bigbite Spectrometer, Experiment at JLab |
| SBU | Stony Brook University |
| SC | Super Conducting |
| SLAC | Stanford Linear Accelerator Center (National Laboratory) |
| SPACAL | Spaghetti Calorimeter |
| SVT | Silicon Vertex Tracker |
| TDC | Time-to-digital converter |
| TDR | Technical Design Report |
| THGEMS | Thick Gas Electron Multipliers |
| ToF | Time of Flight |
| TPC | Total Project Cost |
| TR | Transition Radiation |
| TRD | Transition Radiation Detector |
| US | United States |
| USP | University of São Paulo, Brazil |
| UV | Ultraviolet |
| WLS | Wave Length Shifter |
| ZDC | Zero Degree Calorimeter |

Configuration Management Plan for the Electron-Ion Collider

# INTRODUCTION

## Purpose and Scope

This document details an estimate of the R&D needs for a general-purpose EIC detector. This entails cost and timelines for R&D of all subsystems and technologies that are included in the reference detector, a as well as those that were identified as alternatives. While the reference detector is optimized for the EIC physics with subdetectors based on the most likely technologies, the final concept can be only defined once collaborations are formed.

October 2024 Note: The document remains as written in 2021 to ensure there is a record without a change of history. The original document was written before a detector proposal selection process that was held in 2021/2022, before the formation of the electron-Proton/Ion Collider (ePIC) collaboration was formed in 2022, and before technical baselining of the various ePIC detector components, achieved in 2023/2024. In addition, the broad and international EIC User Group community and the subsequent ePIC collaboration members had to familiarize themselves with US-based projects. As one notable example, in some of the original community-submitted milestones to the detector R&D plan, one will see wording such as “final design”, which is not an R&D but rather a Project Engineering and Design (PED) activity. We will keep the original detector R&D document for its historical value, and then add an “Update in October 2024” section in the end to recap where we are as compared to the technical R&D completion milestones.

## Introduction

The need for R&D was realized early by the community and laboratories and in January 2011 Brookhaven National Laboratory, in association with Jefferson Lab and the DOE Office of Nuclear Physics, created a generic detector R&D program to address the scientific requirements for measurements at an EIC. The primary goals of this program were to develop detector concepts and technologies that have particular importance to experiments in an EIC environment, and to help ensure that the techniques and resources for implementing these technologies are well established within the EIC user community. It was also meant to stimulate the formation of user groups and collaborations that will be essential for the ultimate design effort and construction of the EIC experiments.

Many of the supported projects, ongoing or completed, developed technologies that are now integral parts of the reference detector or are regarded as potential alternatives. R&D on many of the technologies to implement a successful comprehensive are close to completion but will require further funding to achieve the maturity required to carry out preliminary and final design and eventually construction of the project scope elements. I many cases small prototypes exist that are not sufficient to demonstrate the required homogeneity and stability. In these cases, larger prototypes consisting of full chains (detector, electronics, supplies, and DAQ) are needed. These prototypes are typically segments or sectors of larger components, e.g. a TPC sector, a single ladder of an inner Si-vertex tracker, or a block of 4 calorimeter towers. Test beams at various facilities (FNAL, JLAB, CERN) or bench tests with X-rays or radioactive probes are then used to establish their functionality, performance, and reliability.

In what follows we discuss the R&D needs for the reference detector grouped by their purpose: Tracking, particle identification (PID), electromagnetic and hadronic calorimetry, auxiliary detectors, and electronic. At the end we discuss a set of R&D projects that are generic in nature and are not part of the reference detector. This entails more future-looking detector concepts and technologies that have the potential to enhance the scope of EIC science in the outyears or even could serve as potential alternatives.

The information for this document is based on information and planning documents provided by the scientists and engineers that are involved in R&D on the various detector subsystem. Cost and schedule were adjusted to match the latest timeline and project needs. It should be noted that this document represents only an estimate of the R&D needs as the final detector design and the number and type of subsystem will emerge in 2021/22.

# R&D for Reference Detector

## Tracking

### Vertex Tracking

The EIC requires precision tracking with very low mass. The combination of very high single point spatial resolution and very low mass detector layers makes MAPS/DMAPS technology the most suitable candidate. While the current state-of-the-art ALPIDE chip does not meet all EIC requirements, it could do so after moderate modification.

The vertex tracker will consist of barrel (layers) and forward/backward (disks). The total Si-Vertex related R&D includes parts of the sensor development, the sensor modulization, the development of staves for the barrel tracker, disks development for the forward and backward tracker, cooling, service reduction, and integration.

The inner vertexing layers will be quite similar to the ALICE inner tracker ITS/ITS3. Much of the mechanics, cooling, and infrastructure can be simply modified for the EIC use case. This also includes the RDO. The scope of the R&D is to produce and test preliminary prototypes that meet the requirements of a working detector system and to demonstrate that the detector concepts are possible.

Strategy on the sensor side is to (i) development of a EIC specific MAPS/DMAPS sensor, leveraging on the ITS3 effort at CERN (see 2.1) but have (ii) the existing ALPIDE chip as viable backup solution.

The following outlines the timeline for the complete vertex tracker including service reduction (see section 1.1.2) and sensor development (see generic R&D section 2.1).

Timeline:

| Year | Task |
| --- | --- |
| 2021 | • Testing and characterization of MLR1 (Multi-Layer Reticle) • Sensor design for MLR2 • MLR2 submission • R&D into powering, stave/disc construction, cooling, overall infrastructure |
| 2022 | • Testing and characterization of MLR2 • Sensor design for ITS3 ER1 • ITS3 ER1 submission • R&D + prototyping into powering, stave/disc construction, cooling, overall infrastructure |
| 2023 | • Testing and characterization of ITS3 ER1 and assessment of yield • Assessment and planning for EIC sensor fork of ITS3 design • Fork off sensor design and work on EIC variant for staves and discs (may move to next year depending on results) • ER submission for EIC variant sensor (EIC ER1) for staves and discs • Detailed prototyping into powering, stave/disc construction, cooling, overall infrastructure • Investigation of adaptation of ITS3 design for use in EIC vertex layers (different radii, # layers, services from both ends to meet length requirements, etc.) with ITS ER1 |
| 2024 | • Testing and characterization of EIC ER1 and assessment of yield • Si design for EIC ER2 • EIC ER2 submission for EIC variant sensor for staves and discs • Detailed prototyping into powering, stave/disc construction, cooling, overall infrastructure using EIC ER1 prototypes • Adaptation of ITS3 design for use in EIC inner layers with ITS2 ER2 (or integration of design into EIC ER2 if necessary). |
| 2025 | • Testing and characterization of EIC ER2 and assessment of yield • Complete stave and disks prototypes with EIC ER2 • Vertex layers prototypes with ITS2 ER3 |

### Service Reduction

It is critical for the EIC to minimize the number of services to and from the various subdetectors to minimize their respective material and thus reduce multiple-scattering and increase the resolution.  In the context of service reduction, R&D will be needed in radiation tolerant multiplexing (probably using radiation tolerant FPGAs) and in high speed (5 GHz and above) fiber or multi-fiber optical transmission components. Both of these technologies are complimentary and urgent. Furthermore, investigating the possibilities of serial powering, possibly with on chip regulation and the use of on detector radiation tolerant DC-DC converters could reduce the material from power cables substantially. In general, the application of this type of R&D benefits most when it is co-developed with the detector technology (MAPS sensors, GEMS, etc.). This R&D should be considered as part of the system level approach to developing detector solutions. This is envisioned for a day one detector implementation.

Timeline (see section 1.1.1)

### Central Tracker TPC

The R&D of a TPC focuses solely on the read-out chambers. Gating grids that have been used in TPCs based on MWPC cannot be used in an EIC environment. The readout rate would not allow to cope with the luminosity requirement of the physics program. The obvious choice are readout chambers using micro-pattern gas detector (MPGD) technologies. This requires the investigation of MPGD-properties and different gas choices which find the optimum of relatively good ion back flow (IBF) suppression and optimum dE/dx resolution for charged particle identification (PID).

The Micromegas technology has the best intrinsic IBF suppression and is a good candidate for good dE/dx resolution. However, stability issues have to be investigated and is an indicator for R&D in the next time for pursuing the Micromegas option.  A very promising candidate for combining very good IBF suppression and good energy resolution is the hybrid option of combining Micromegas and GEMs into a single amplification stage. The Micromegas acts as the main amplification stage and reduces the IBF to a minimum. The GEMs act as pre-amplifiers and provide the necessary field ratios to further suppress IBF. The combination of both technologies provides the robustness needed to operate in a high-rate environment. This amplification structure needs continued detailed investigation.

Even if the sPHENIX TPC is reused, new readout chambers will have to be installed since the sPHENIX TPC has been optimized to minimize the ion back flow (IBF) suppression sacrificing good dE/dx resolution. For the EIC program this feature has to be restored.

Timeline:

| Year | Task |
| --- | --- |
| 2021 | Continue testing readout options (4 GEM vs 2 GEM + MMG, zigzag: charge sharing) using small scale prototype. Provide input on TPC design for hybrid detector design with TPC and Si inner tracker. ln parallel, continue to investigate applicability of interleaved readout planes for a planar tracker option. |
| 2022 | Design and build prototype hybrid tracker and test in beam. Continue with engineering design and simulations for full scale detector. |
| 2023 | Revise engineering design based on results of prototype tests and design full scale prototype. |
| 2024 | Build full scale prototype and test in test beam. Revise final design based on prototype tests. |
| 2025 | Complete final design and prepare for construction. |

### Low-Mass Forward/Backward GEM Tracker

Gas Electron Multipliers (GEMs) are a well-established MPGD detector technology that will soon be operational on a large scale in current NP and HEP experiments, e.g., SBS tracker, ALICE TPC upgrade, and CMS muon upgrade. In a day-one EIC detector, they do provide cost-efficient fast tracking with good spatial resolution in the forward and backward regions because they can cover a large area.

The plan is to complete characterization of the two large GEM prototypes (UVa & FIT) by December 2021. To bring low-mass GEM tracker technology to a state where it can be implemented in an EIC detector some modest R&D is still required. This includes improvements in the simulations and a second test beam.

The simulations need to be repeated and refined. The actually measured spatial resolutions and realistic support materials need to be incorporated properly into the simulation, in particular the materials in the TPC endplates, MAPS support structures, and the GEM support frames. Their impacts on forward/backward tracking performance and RICH seeding need to be fully quantified. This should take six months to a year to complete.

For the glued prototype at UVa different types of zebra strip connectorizations need to be tested. The mechanically stretched FIT prototype with carbon fiber frames has been undergoing major refurbishments of it mechanics and its operation needs to be confirmed. If successful, both prototypes will be evaluated in a second beam test at Fermilab planned for Summer 2021 to finalize the spatial resolution studies and the overall performance characterization of the prototypes.

Timeline:

| Year | Task |
| --- | --- |
| 2021 | June: Test performance in beam test at FNAL December: Finalize FNAL test beam data analysis |
| 2022 | July: Completion of R&D program |

## Particle Identification

### Modular RICH (mRICH)

mRICH stands for compact and modular Ring Imaging CHerenkov detector, which is designed for K/𝜋 separation in a momentum range of 3 to 10 GeV/c and e/𝜋 separation below 2 GeV/c for the future EIC experiments.

The key components of a mRICH module include an aerogel radiator, a Fresnel lens, a mirror set, and a photosensor. A realistic GEANT4-based simulation for mRICH has also been developed and verified with beam test data. Two rounds of detector prototyping and beam tests were completed with a focus on verifying the detector working principle and performance. The second beam test was done in 2018 and the data analysis is still ongoing. Two more beam tests with particle tracking capability, crucial to evaluate the Cherenkov single photon resolution, are under preparation in order to quantify the mRICH PID performance and new photosensors. One was originally planned for March ’21 but is now postponed due to the COVID pandemic. Attempts are under way to have test beams beginning of June ’21 (May 25 to June 15) for testing the mRICH also with a LAPPD. The other test is planned at JLab in summer of 2021 using secondary electrons in momentum range from 1 to 6 GeV/$c.

To meet the needs of EIC experiments, a proper photosensor choice is critical. The planned beam test at Fermilab in May/June ‘21 (pending DOE travel approval to FNAL) will help to evaluate the integration and performance with LAPPD. During the second mRICH beam test in 2018, three SiPM matrices were tested with varying cooling temperature range from -30 C degree to room temperature. The radiation damage effects to SiPM performance is currently under study at INFN.

In regarding to the possible kinematic coverage in EIC experiments with mRICH modules, one can envision deployment in the electron endcap and the hadron endcap (1 < 𝜂 < 2.5). The mRICH is considered as a day-1 detector.

Besides the two planned mRICH beam tests in the coming year, there are R&D efforts towards engineering design which includes: (i) high quality mirror and mirror assembly, (ii) mRICH holder box engineering for reducing total weight, easy assembling, and projective installation, and (iii) continued test with available photosensor options.

Timeline:

Note: In the timeline above the light gray color in the progress bars indicate optional activities while the dark gray stands for planned/required activities.

### Dual-Radiator Ring Imaging Cherenkov Detector (dRICH)

The dual-radiator Ring Imaging Cherenkov (dRICH) detector is designed to provide continuous full hadron identification (𝜋/K/p) separation better than 3𝜎 from 3 GeV/c to 50 GeV/c in the forward range. It also offers a electron and positron identification (e/𝜋 separation) from few hundred MeV up to about 15 GeV/c. Achieving such a momentum coverage in the forward ion-side region is a key requirement for the EIC physics program.

A small-scale prototype is being developed to investigate critical aspects of the proposed dRICH detector, in particular related to the interplay and long-term performance of the two radiators and the simultaneous imaging. The prototype vessel is composed by standard vacuum parts to contain the cost and support pressures different from the atmospheric one. This would allow efficient gas exchange and, in principle, adjustment of the refractive index and consequent flexibility in the gas choice (in the search for alternatives to greenhouse gases). The prototype supports the usage of various type of photosensors, in particular SiPM matrices and MCP-PMTs.

A program has been planned to study the potential of SiPM sensors for Cherenkov applications, aiming to an assessment of the use of irradiated SiPM in conjunction with the dRICH prototype. The SiPM response before and after irradiation will be characterized and their imaging potential will be studied with customized electronics. High-frequency sampling and Time-of-Threshold-based readouts will be compared. Of particular interest is the ALCOR front-end chip designed to work down to cryogenics temperatures. The irradiated sensors will be cooled down to the working temperature (down to -40 C) to instrument an area suitable for imaging tests with the dRICH prototype. After an initial survey of the most promising candidates available on the market, a dedicated R&D needs to be pursued to meet the EIC specifications. Despite the tight schedule, the group is on track for the tests scheduled in mid-May. They have acquired all the sensors and produced the relative SiPM carrier boards,  and the test plan for the SiPM characterization as well as for the administrative controls, is being developed.

Besides the first SiPM irradiation campaign and the baseline prototype realization, there is a need for R&D efforts towards engineering design which includes: (i) light and stiff support structure in composite materials, (ii) a high-quality mirror assembly and (iii)), the cost-effective production of high-quality aerogel, (iv) the study of alternatives to the originally envisioned greenhouse gases, and (f) dedicated readout electronics and cooling.

The timeline for “sensors and electronics” in the table below refers to the "Collaboration effort within eRD14" that comprises synergy developments and alternative technology comparison. The activity foreseen for 2024 (not included in the table) is meant to finalize dRICH dedicated solutions, in particular regarding engineering and integration into EIC. In any case, the table presents an aggressive plan assuming an adequate funds availability, with manpower support being the most critical.

Timeline:

### High-Performance DIRC (hpDIRC)

The high-performance DIRC (hpDIRC) is a proposed hadronic PID system for the barrel region of the central detector, capable of 𝜋/K separation with 3𝜎 or more up to at least 6 GeV/c momentum over a wide angular range. It can also contribute to e/𝜋 identification at lower momenta.

The R&D of the hpDIRC is at an advanced stage. It has low demands on the detector infrastructure (no cryogenic cooling, no flammable gases) and is easy to operate. The PID performance estimate is based on test beam results, with excellent agreement between simulation and prototype data.

Several areas still require significant R&D. Optimizing the cost-efficient design, matched to the final EIC detector layout, in simulation and validating it with the full system hpDIRC prototype is the most critical item. R&D will be needed to develope a procedure to disassemble the BaBar DIRC bar boxes and extract high-quality radiator bars for hpDIRC. These tests will happen at SLAC with local support. If possible, these test could be even earlier. The hpDIRC performance does not depend much on the type of bar, if they are reused from BaBar or brand new (plan B) - as long as the optical and mechanical quality of the disassembled bars is good. The main impact of a negative outcome of the reuse R&D would, therefore, be financial. There used to be a significant schedule risk associated with the mass production of new bars but the recent success of the bar production for PANDA has shown that this risk can be assumed to be only minor for the EIC detector as well.

Support is needed soon to upgrade the PANDA DIRC prototype, which is being transferred from GSI to CUA/SBU, to fully equip it with new sensors and electronics, in order to validate the resolution and PID performance with cosmic muons and/or particle beams. A new Cosmic Ray Telescope (CRT) facility is being developed for the hpDIRC in collaboration between SBU, ODU, and CUA to study the prototype prior to possible tests in particle beams. This CRT will be available for use by other EIC systems.

Note that the MCP-PMTs for the prototype test require substantial funding (~$200k) in FY22. These sensors can be later used in the actual hpDIRC detector. They will be also used for mRICH (see Sec. 1.2.1) and dRICH (see Sec. 1.2.2) prototype tests.

Timeline:

### Fast Timing Silicon Sensor: LGADs

The Low Gain Avalanche Detector (LGAD) is an ultra-fast silicon sensor technology, which has recently been chosen for constructing a fast-timing layer in the forward rapidity region of CMS and ATLAS for high-luminosity LHC starting in 2027. LGAD sensors can achieve a typical time resolution of about 30 ps. With excellent timing and position resolutions, the LGADs provide an attractive option for constructing a ToF system and are the technology of choice for far-forward hadron spectrometer (Roman Pots). In addition, the LGADs have several other key advantages such as being highly tolerant to strong magnetic fields, radiation-hardness, and compactness.

While LGAD silicon sensors used by CMS and ATLAS can provide a time resolution of 30-50 ps, particle flight distance at an EIC detector is much shorter due to tight space constraints. Therefore, a total time resolution (including readout electronics) of 20 ps or better per layer is needed. Improvements can be achieved by reducing the thickness of the sensor and optimizing internal and external gains. Resolutions of 20-25 ps are expected to be feasible. To serve as (part of) a tracking system as is the case for the far forward spectrometer and ToF/tracker, a position resolution better than the 1 mm pixel size has to be accomplished. To achieve this two solutions exist: Trench-isolated LGADs and AC-coupled LGADs where the latter is seen as preferrable for the EIC. R&D is needed to fully characterize the AC-LGAD performance, test their robustness and optimize their design for the specific implementation in Roman Pots and possibly ToF detectors. The intrinsic sensor gain and thickness needs to be optimized to improve the time resolution, finer spatial resolution can be achieved by exploiting the signal sharing properties of neighboring pixels, and larger area prototypes with advanced designs need to be fabricated and tested.

The needs for better timing performance and finer granularity also pose significant challenges to the readout electronics and specifically to the ASIC readout chips. Most critical at this point in time is the development of an architecture of the readout electronics, and more urgently the ASIC R&D.

Timeline:

In a timeframe of 2 years, thanks to prototyping and laboratory testing, the AC-LGAD can be confirmed as the baseline technology for Roman Pots, while an optimization of the sensor readout can be achieved in a 5-year time scale.  In a 2-year timeframe the readout architecture can be developed, and its viability demonstrated via simulations as well as laboratory tests based on existing prototypes for the LHC, while in a 5-year time scale a more detailed design of the ASICs and the readout chain, including initial prototyping, can be achieved.

## Electromagnetic Calorimetry

### Scintillating Glass & Crystals

The requirement of high-precision detection of electrons at forward rapidities is driven mainly by inclusive DIS where the scattered electron is critical for all processes to determine the event kinematics. Excellent electromagnetic calorimeter resolution of better than 2%/E is required at small scattering angles. For hadron physics measurements with electromagnetic reactions, the most common precision calorimeter material of choice has been lead tungstate, PbWO4. However, the production of crystals is slow and expensive.

The technology goal of SciGlass R&D is to develop a scintillating glass for homogeneous electromagnetic calorimetry. SciGlass fabrication is expected to be cheaper, faster, and more flexible than PbWO4 crystals. SciGlass is being developed by Scintilex, LLC in collaboration with the Vitreous State Laboratory at CUA. Tremendous progress has been made in the formulation and production of SciGlass that improves properties and solves the issue of macro defects. Scintilex has demonstrated a successful scaleup method and can now reliably produce glass samples of sizes up to 10 radiation lengths. Simulations combined with initial beam tests at photon energies of 4-5 GeV suggest that high resolution competitive with PbWO4 can be reached.

The areas of needed R&D for SciGlass include the final formulation optimization, scale up to block sizes  ≳15 X0 (most critical item) and beam tests to establish characteristics like energy resolution. The evaluation of SciGlass as particle detector has been shared in part with activities on PbWO4 crystals for the electron endcap calorimeter, including simulations, radiator characterization and prototype construction, commissioning, and beam tests. The approximate timeline for completing the SciGlass R&D is about 1.5 years. The goal is to be ready for a day-1 detector. SciGlass could also be available for future detector upgrades.

Timeline:

| Year | Task |  |
| --- | --- | --- |
|  | PbWO4 | SciGlass |
| 2021 | Complete prototype tests with different readout options | Final formulation optimization, scale up to block sizes ≥15 X0, and establish SciGlass characteristics. |
| 2022 |  | Prototype and beam tests. Process design verification to scale up. |

### W powder SciFi, Pb(W)/Sci Shashlik, and Lead Glass

An EIC detector does require a hermetic coverage (𝜂 < |3.5|) of electromagnetic calorimetry with varying requirements depending on its 𝜂 range. Various technologies are available, all requiring R&D to optimize the light collection and resolution. The construction of a full chain prototype to verify and test the performance parameters are highly desired.

Tungsten scintillator calorimetry can play a major role in many of the regions of an EIC detector. It offers a very compact design in terms of its short radiation length as well as providing a small Moliere radius which limits the lateral extent of the shower, therefore allowing good separation between neighboring electromagnetic showers. There are primarily two candidates that are being considering for a W/Scint calorimeter for EIC. One is a tungsten scintillating fiber (W/SciFi) SPACAL, which consists of a matrix of tungsten powder and epoxy with embedded scintillating fibers. The blocks are read out using SiPMs that are coupled to the blocks using short light guides.

No R&D is required for producing the blocks. However, the method used for reading out the blocks with SiPMs needs to be improved. This would include the use of large area SiPMs to provide more photocathode coverage and eliminate the boundaries between the light guides which leads to non-uniformities in the energy response.

The second W/Scint technology that is being considered is a tungsten shashlik (W/Shashlik) design. A W/Shashlik design offers some distinct advantages but also poses some significant challenges. In addition to being compact and being able to tune the energy resolution as in the W/SciFi, a W/Shashlik offers the possibility of improving the light collection and providing better uniformity by reading out each individual WLS fiber with its own SiPM. Also here, R&D is need to optimize the light collection and resolution as well as full-chain prototype.

Other technologies are considered such as lead glass, which has the advantage that many lead glass blocks exist and could be reused. Same R&D needs as for W/SciFi and W/Shashlik applies.

Timeline:

| Year | Task |  |
| --- | --- | --- |
|  | W/SciFi | W/Shashlik |
| 2021 | Continue testing readout options to increase photocathode coverage. Design small prototype (4x4 blocks) to test actual performance. Begin engineering design(s) for actual detector. Barrel and Endcap would require completely different designs. | Design medium size (~ 25 cm2) prototype. Carry out simulations for expected performance. Develop method for making modules projective. Begin engineering design(s) for actual detector. Barrel and Endcap would require completely different designs. |
| 2022 | Build small prototype and test in test beam. Continue with engineering design(s) for full scale detector. | Build prototype(s) and test in test beam. Will likely require at least 2 iterations. Barrel and Endcap would require separate prototypes. Continue with simulations and engineering designs. Design of readout electronics would need to go on in parallel. |
| 2023 | Revise engineering design(s) based on results of prototype tests and design full scale prototype(s). |  |
| 2024 | Build full scale prototype(s). Test in test beam if possible. | Revise engineering design(s) based on results of prototype tests. Design full scale prototype(s). |
| 2025 | Complete final design and prepare for construction. | Build full scale prototype(s). Test in test beam if possible. |
| 2026 |  | Complete final design and prepare for construction. |

## Hadron Calorimetry

The requirements for the resolution of the hadronic calorimeter are different for the endcaps and the barrel region. The most challenging is the forward region of hadronic endcap where pure calorimetric measurements starts to outperform particle-flow like approaches due to the degradation of tracker performance.  For the electron endcap and the barrel region, only modest hadronic energy resolution is required.

It is believed that the barrel HCAL can be built using standard construction methods and no additional R&D efforts other than the construction and testing of a full chain prototype is needed.

For the hadronic endcap, where better energy resolution is required, modest R&D efforts will be needed to improve the performance of these systems.

At more forward rapidities in the hadron endcap the requirements on energy resolution become more stringent. Here it is important to have the best possible performance of the calorimeter system.  The main constrain is the lack of space for a high sampling fraction and high sampling frequency calorimetry system, both of which are required to achieve good resolution. Developing a high-resolution calorimetry system for this region will require significant R&D efforts. At present we believe that there is only one technology option that may be suitable for this region, which is a very high density, approximately compensated fiber calorimeter, which could serve as both the EMCAL and HCAL with a common readout.

The readout with SiPM sensors may be challenging at the forward rapidities of the hadron endcap due to the relatively low light yield of hadron calorimeters (compared to EM calorimeters), and the high neutron fluences in this region, which will lead to significant degradations in SiPM performance. Future R&D is therefore needed in this direction.

Timeline:

| Year | Task |
| --- | --- |
| 2021 | Finish optimization of light collection for HCal (WLS/SiPMs). Work out tile catcher integration and frontend electronics for Fe/Sc HCAl Preparation for the construction (need authorization to order long lead time materials. Start construction of 0.6 x 0.6 WScFi + Fe/Sc hadron endcap |
| 2022 | Completion of construction of a full-scale hadron endcap prototype Beam testing at FNAL in the fall if beam available |
| 2023 | More beam testing if needed, Update TDR |

## Auxiliary Detectors

### Low Q2 Tagger

The low-Q2 taggers consist of a set of small-scale trackers and an electromagnetic calorimeter to capture angle and energy/p of the scattered electron ~15m away from the IP. They will be finely segmented to disentangle the multiple electron hits per bunch crossing from the high-rate bremsstrahlung process. The taggers need to have position sensitive detectors to measure the vertical and horizontal coordinates of electrons. The combined energy and position measurements allow reconstruction of the key kinematic variables. If the position detectors have multiple layers and are able to reconstruct the electron direction this will over constrain the variable reconstruction and improve their measurement; this may also provide some measure of background rejection. There will provide resolution for Q2 as low as 10-3 GeV2 in the range accepting -6.9 < 𝜂 < -5.8.

No R&D is needed for a low-Q2 tagger, except the construction of a prototype and verification of its performance in test beams.

Timeline: not critical

### Roman Pots

A far-forward proton/ion spectrometer, based on the well-known technique of Roman Pots, is an integral part of an EIC detector system, essential for the success of its physics program and thus is envisioned as a subsystem for a day-one EIC detector.

An innovative silicon technology, based on AC-LGAD, is proposed to instrument the Roman Pots, which has the potential to combine in a single sensor fine spatial resolution and precise timing. The development and optimization of the sensors is discussed in Sec. 1.2.4.

The adaption of the sensors and corresponding FEE (ASICS) and optimization for the Roman Pots will require R&D. In addition, a larger area prototypes with advanced designs need to be fabricated and tested.

Timeline (see 1.2.4)

### Zero Degree Calorimeter (ZDC)

The ZDC will serve critical roles for a number of important physics topics at EIC, such as distinguishing between coherent diffractive scattering in which the nucleus remains intact, and incoherent scattering in which the nucleus breaks up, measuring geometry of e+A collisions, spectator tagging in e +D/ 3He, asymmetries of leading baryons, and spectroscopy.

These physics goals require that the ZDCs have high efficiency for neutrons and for low-energy photons, excellent energy, pT and position resolutions, large acceptance and sufficient radiation hardness.

It is anticipated to be a sampling type calorimeter with a sufficient longitudinal size of∼10 interaction length. It is also required to have a sufficient transverse size of∼2 interaction length to avoid transverse leakage of the hadron shower and to achieve good hadron energy resolution. There are various technologies available, such as the one used for the FOCAL calorimeter in ALICE, Cherenkov calorimeters, as well as technologies envisioned already for the EIC like crystals, scintillating glasses, and W/SciFi calorimetry. Cerenkov calorimeters, which measure only the high energy component of the showers, give excellent position resolution and tight containment but are non-compensating and thus non-linear. Sampling all charged particles produced, gives better energy resolution at the cost of worse lateral containment.  For the ZDC one will need to exploit both techniques to maximize both the energy and position resolution as was shown be possible using quartz fibers developed or the LHC ZDCs.

For the ZDC much use can be made of past R&D. ZDC specific R&D is therefore not needed, except the construction of a full chain small to moderate prototype and test beam.

Timeline: not critical

## Electronics

Substantial R&D will be need for the development of Front-End Electronics (FEE). Here we define FEE as ASIC, Front-End Board (FEB), and Front-End Processor (FEP). FEPs are needed if the ASIC does not provide all features needed. The choice of using streaming read-out for the EIC excludes several existing ASIC chips. Although one can possibly run ASICs not intended to operate in streaming mode, in a way that emulates such with local trigger or in self-triggered mode, it brings with it a high risk, especially in the presence of background which increases the deadtime and inefficiency to unacceptable levels.

ASICs exist for the Si-Vertex (DMAPS) detector, since the current ALICE ITS ships meet EIC requirements. ASICs for the readout of LGADs/AC-LGADS need to be urgently developed but their development is already contained in the LGAD R&D needs as discussed in Sec. 1.2.4.

We estimate the need for 3 ASICs used for the readout of (i) SiPM (calorimetry), (ii) MCP-PMT/PMT (PID), and MMG/GEM2/𝜇RWell (tracking). This will require 3 FEB and likely 1-2 FEP.

Timeline: ASIC development takes 4-5 years. Considering the project deadlines (CD-1 to CD-4), developments of the various parts will likely have to occur concurrently, so multiple groups will have to be involved. Final requirements can only be established once the detector technologies are finalized (Q1 2022) but should not prevent first R&D efforts.

# Generic R&D for Alternative Technologies

Here we list various alternative technologies that are not part of the reference detector but are potential candidates. Some will require substantial generic R&D to reach maturity.

## Development of MAPS/DMAPS Sensor for the EIC

The goal of this R&D is to develop sensors that meet the stringent EIC requirements for vertexing and tracking. The combination of very high single point spatial resolution and very low mass detector layers makes MAPS/DMAPS technology the most suitable candidate. The needed R&D is in parts to support the development of a MAPS sensor based on the ITS3 effort currently underway at CERN. The goal of this consortium is to develop a MAPS sensor and associated powering, support structures, control and ancillary parts as necessary to produce a detector solution for silicon tracking and vertexing for the central tracking parts of an EIC detector. This will include 1 MLR run shared with ITS3 (FY22), 1 engineering run shared with ITS3 (FY23), and 2 engineering runs for EIC chips only. Each of the runs is followed by a phase of intensive testing and characterization of the produced chips

It is estimated that 80% FTEs for silicon development would likely come from members of the EIC silicon consortium with support from their funding agencies. This would include DOE for US institutions with the contribution presumably on their base grant as well as foreign contribution funded from their host agencies.

For complete timeline (see section 1.1.1)

Timeline (short version):

| Year | Task |
| --- | --- |
| 2021 | Submission of the second MLR |
| 2022 | Submission of the first engineering run (ITS) |
| 2023 | Submission of the first engineering run (EIC variant), second engineering run (ITS3). |
| 2024 | Submission of the second engineering run (EIC variant). |
| 2025 | Integration of prototype sensors into disc and stave. Possible contingency submission of EIC variant. |

## Central Tracker Cylindrical µRWELL

One significant need for large cylindrical 𝜇RWELL layers in the central barrel region is to provide high angular resolution for barrel PID detectors. This additional tracking information can aid in the PID particle seed reconstruction leading to better particle separation. For the scenario where a TPC is chosen as the central tracker option for the EIC detector a cylindrical 𝜇RWELL layer serves as a high space point resolution tracking layer to aid in the TPC field distortion corrections and TPC calibrations.

The simple construction of a 𝜇RWELL detector relative to a triple-GEM detector makes it an ideal MPGD technology to use in a cylindrical geometry. There are several R&D items related to its construction and performance that still need to be investigated. The first is related to the 𝜇RWELL technology itself. Efforts are needed to reduce the overall material budget of the current "standard" 𝜇RWELL. This involves the development of low mass amplification and readout structures. Ideally the cylindrical 𝜇RWELL would consist of one large foil and thus have no dead region in the active area. However, like with GEMs, 𝜇RWELL raw material is limited to a width of about 50 cm. To provide proper coverage for a barrel PID detector, several 𝜇RWELL will be needed to form the full cylindrical layer. R&D is needed to determine best way to integrate the 𝜇RWELL into one large cylindrical detector while minimizing dead regions in the active area. Another area of R&D that is needed is related to the support structure of the cylindrical 𝜇RWELL layer. This involves developing large, high strength and lightweight cylindrical 𝜇RWELL supports to hold the detector's cylindrical shape. Additionally, end cap structures to hold the cylindrical detector in place need to be designed. Several performance studies such as rate capabilities, dE/dx, tracking, and timing resolutions need to be carried out. The detector's cylindrical uniformity, discharge rate and aging properties will also need to be assessed.

Timeline:

| Year | Task |
| --- | --- |
| 2021 | Completion of mechanical mockup. Procure components and begin building small-radius functional cylindrical prototype. |
| 2022 | Complete construction of small cylindrical prototype. Commission prototype. Perform beam test of small cylindrical prototype. Analyze beam test results. Design and procure materials for full-size mechanical mock-up. |
| 2023 | Finish analyzing beam test results. Build full-size mechanical mock-up ad evaluate. Design and build large-radius cylindrical 𝜇RWell prototype. |
| 2024 | Complete large prototype. Perform beam tests with large prototype. |
| 2025 | Complete analysis from test beam of large cylindrical 𝜇RWell prototype. Ready for design of potential production detectors. |

## Central Tracker Cylindrical Micromegas

Large cylindrical Micromegas layers in the central barrel region serve the same purpose as described in Sec. 2.2. Micromegas detectors have been already successfully employed for building compact and light trackers in various experiment (e.g. CLAS12). R&D is therefore limited to reducing the material budget (minimizing X0), readout adaption and optimization and the construction of a full chain prototype.

On most MPGDs, copper is the chosen readout material with a thickness of at least 9 𝜇m. The use of lower mass material for the strip readout such as metalized aluminum of about 0.4 𝜇m requires R&D. The aluminum strips will have to be protected by a resistive layer to prevent vaporization of the metalized layer due to sparks.

Standard connectors made of plastic and brass contacts are quite heavy in term of material budget. If the active area is segmented, the multiplication of connectors can be a problem. Further R&D is need for testing Kapton connections with metal pixels clamped with light materials (carbon or 3D printed plastic).

Timeline:

| Year | Task |
| --- | --- |
| 2021 | Ultra-light: R&D Goal: from 0.5% X0 (Clas12) to 0.05% X0 through R&D Full simulation of ultra-light MM design Design and construction of stretch bulked Kapton demonstrator (no FR4) 2D readout design studies: Procurement of large pads readout PCBs Finalize 2D zigzag readout pattern studies |
| 2022 | Ultra-light: Aluminium based strips Thin aluminium mesh manufacturing with laser ablation Readout studies: Bulking and test of large pads readout Cylindrical MM: Design of MM tracker support structure within EIC detector |
| 2023 | Ultra-light: Prototype construction Cylindrical MM: Final prototype with 2D zigzag readout |

## GEM-TRD/Tracker

A high granularity tracker combined with a transition radiation option for particle identification would provide additional information necessary for electron identification or hadron suppression. The scope of the project is to develop a transition radiation detector/tracker capable of providing additional pion rejection (> 10-100).

A low mass radiator available for mass production is critical and various materials still need to be tested and optimized. This includes the optimization of a pseudo-regular radiator using thin Kapton foils and thin net spacers and a detailed test of available fleece/foam materials for TR-yield.  The transition radiation detector readout is based on well-established GEM technology but features a thick drift volume. In order to keep a uniform electric field, a special field cage needs to be developed. This includes the mechanical design and construction of the field-/gas-cage to minimize a Xe-filled gas gap between radiator and the drift cathode. The anode readout PCB layer of the current GEM-TRD prototype is based on a readout developed for the COMPASS experiment. While this is optimal for a high occupancy environment, the large number of channels does increase the price of the readout electronics. R&D is needed way to develop a new concept of pad readout better suited for GEM-TRD applications.

The use of existing readout chips (SAMPA) that meet the requirements will need to be evaluated in detail. Development of a new readout chip will be needed to enable the streaming of zero-suppressed data over fiber links.

Over the past few years, the price of Xe has gone up significantly. Design and Development of a recirculation system to purify, distribute, circulate, and recover the gas, possibly based on a design of ATLAS TRD gas system at CERN will be necessary but will require only moderate R&D.

Timeline:

| Year | Task |
| --- | --- |
| 2021 | - Test of different readout architecture ( strips, pad,zig-zag) to minimize the noise level, number of readout channels, and spatial resolution. This requires building several small (10x10 cm2) prototypes with different readouts options and test them at JLAB and Fermilab. - Tests of a new streaming readout architecture hardware (SRO125) and ML-FPGA -based data reduction concepts. - Test of different TR-radiators. |
| 2022 | - Build and test large-size modules in order be able to workout possible issues: like noise, gain-uniformity, drift-time issues, HV stability, etc. A field/gas-cage needs to be developed and optimized for TRD applications. - Test beams at Fermilab with electron and pion beams |
| 2023 | - Design and development of a recirculation gas system to purify, distribute, circulate, and recover the gas (in collaboration with other labs/universities). - Test beams at Fermilab with electron and pion beams |
| 2024 | - Development of final design specifications for the streaming readout architecture as input to a coordinated ASIC design program. |

## Time-Of-Flight Detector (ToF)

Two time-of-flight detector techniques are under consideration. One is using AC-LGAD precision timing silicon detectors; the other option is making use of the excellent timing performance of LAPPDs. Due to the compactness of the EIC detector, flight paths are short and resolutions in the order of ~10-20 ps will be needed. It is expected that AC-LGAD will be able to reach resolution if 20-35 ps and LAPPDs of ~10 ps.

Another option that has not received much attention yet but might have quite some potential is the technology used in the Barrel Timing Later (BLT) of the CMS-MTD design, i.e. using crystals with SiPM readout. Test beams confirm resolutions of 30ps that would fill in the PID gap with 3𝜎 separation for particles with p < 2-3 GeV/c

ToF are considered in the forward direction to enhance PID, and in the barrel in the case of an all-Si tracker (instead of a TPC) to compensate the loss of PID provided through the TPC’s dE/dx measurements at low-pT.

R&D efforts focus mostly on the development of LGAD sensors only and are discussed in Sec. 1.2.4 for AC-LGAD and Sec 2.6 for LAPPDs. Note that even if no AC-LGAD based TOF will be chosen for the final detector that sensor R&D part is still needed since it is the technology of choice for the Roman Pots.

Current plans are to follow the CMS-MTD concept. The LGAD technology is being applied to the upgrade of CMS and ATLAS timing layers for the high-luminosity LHC program. Many technical and engineering challenges for constructing and operating a full detector are being addressed there. Therefore, the LGADs is a mature technology that is ready for EIC detectors. At EIC, the detector radiation hardness is not a concern but requirements on the precision of PID and tracking are more demanding, requiring targeted R&D efforts to take advantage of most recent LGADs technologies and develop readout electronics with finer granularity and better power efficiency.

Besides R&D work on the sensors, the priority and potentially challenge is to develop a strategy toward developing the readout electronics needed. ASIC chips developed at CMS and ATLAS can serve as a starting point but more work is needed to optimize the jitter, power consumption and demonstrate the feasibility of reducing the granularity to as small as 500 𝜇m (forward region). Efforts on initial designs of modules, mechanical structure, cooling service, data flow, clock distribution etc. are required for the TDR, by leveraging and optimizing designs at LHC experiments.

Note that this effort is tightly connected to 1.2.4.

Timeline: much in flux. Depends strongly on choice of chosen technologies (LAPPD, LGAD, LYSOL+SiPM).

## Photosensors

The choice of photosensors is essential for reaching the cost and performance goals of all EIC PID subsystems. Solutions for each detector is driven by the detector's specific operational parameters. Ultimately, it would be preferable to use a common photosensor thus reducing development and procurement costs.

Microchannel-plate photomultipliers (MCP-PMTs) from commercial vendors have shown superior good timing and position resolution as well as moderate magnetic field tolerance but are generally far too expensive for large area coverage. The recently commercialized new type of MCP-PMT using the atomic layer deposition technique as a large area picosecond photodetector (LAPPD) provides a promising cost-effective MCP-PMT for the EIC RICH detectors.

R&D efforts using a small format of LAPPD build at Argonne, has demonstrated all the required parameters, especially a magnetic field tolerance over 1.5. To expedite the application of MCP-PMT for EIC Cherenkov detectors, a 10x10 cm2 MCP-PMT fabrication facility is under construction to produce larger size, high-performance MCP-PMTs. The commercial available LAPPD module has also achieved almost all the requirements except fine pixel size and magnetic field tolerance. R&D efforts and close collaboration with INCOM are needed to overcome its shortcoming and produce a viable solution.

To validate the LAPPD performance and apply this new technology to the EIC-PID subsystems, critical R&D is needed in the next two years.  A bench test and multiple beam tests of Cherenkov prototype detectors using the MCP-PMT/LAPPD will need to be performed.

The required R&D is aimed at both near-term and future detector designs

Timeline:

# Generic R&D for Outyears

The R&D needs listed in the previous section are for a day-1 detector. However, further opportunities do remain.  These are driven both by pursuing alternative detector technologies for a complementary second fully integrated EIC detector and Interaction Region, and to prepare for future cost-effective detector upgrades to enhance capabilities addressing new nuclear physics opportunities. Furthermore, the EIC will be a two-decade Nuclear Physics facility after its construction is completed and will in this period likely require further detector upgrades driven by its science findings. It is expected that further physics opportunities enabled by new detector capabilities will already arise during the EIC design and construction phase.

In the remainder of this section, we list possible generic topics of high relevance for an EIC detector.

## MPGD for RICH/Nano-Diamond Materials

Single Photon Detectors (PD) for Cherenkov imaging devices represent a key challenge at EIC where minimum material budget and operation in high magnetic field is required. Gaseous PDs, which have played a major role in establishing and operating Ring Imaging CHerenkov (RICH) counters, satisfy these requirements and they represent the most cost-effective solution when equipping large detector areas. So far, the only photon converter successfully coupled to gaseous detector is CsI with Quantum Efficiency (QE) limited to the far UV domain. Optimized detector architecture and operative conditions have to be established to ensure effective photoelectron extraction. MPGD technologies are considered to be useful in this context.

An R&D program for further developments of an hybrid approach (THGEMS and resistive MICROMEGAS) which is in operation in the COMPASS experiment is desirable. R&D includes: (i) Establishing the hybrid PD for a windowless RICH approach to increase the number of detected Cherenkov photons and (ii) coupling of the THGEMs with a novel and more robust photoconverter made out of Hydrogenated Nano Diamond powder (HND). The latter will overcome the limitation imposed by the use of CsI due to its chemical fragility in contaminated atmosphere or under ion bombardment, that imposes gain limitations and complex handing. Very promising initial studies are already ongoing.

Timeline:

| Year | Development of MPGDs for high momentum hadron PID: Development of MPGD sensors for single photons |
| --- | --- |
| 2021 | Complete construction of prototype version 2 Initial labe test of prototype version 2 Read-out chain based on VMM2 FE fully operational |
| 2022 | Complete lab test of prototype version 2 Validate the read-ou of single photoelectron signals with VMM3 |
| 2023 | If selected for the EIC detector, detailed engineering |

Timeline:

| Year | Development of MPGDs for high momentum hadron PID: New photocathode materials for gaseous detectors |
| --- | --- |
| 2021 | H-ND effective QE in different gas mixtures H-ND effective QE after thermal cycle in inert gas Construction of complete detector with H-ND photocathodes Coating substrate sample with CSI for comparative studies |
| 2022 | Perform comparative studies of H-ND and CsI Lab test of the complete detector with H-ND photocathodes Quantify the radiation hardness of H-ND photocathodes |
| 2023 | Systematic measurements of H-ND with different grain size Systematic measurements of H-ND with different graphite content Systematic measurements of H-ND with different providers Systematic measurements of H-ND with different B doping |
| 2024 | Completion of 2023 exercises |

## CSGlass for Hadronic Calorimetry

There is a need to improve the energy resolution of hadron calorimetry.  The technology goal of CSGlass R&D is to develop a scintillating glass for improving hadronic calorimeter resolution, which is desired or measurements of hadronic jets. CSGlass is optimized for the dual readout approach, where one compares the signals produced by Cherenkov and Scintillation light in the same detector. This approach has been a promising method to achieve better performance for hadron calorimeters.

The areas of needed R&D for CSGlass include the demonstration of CSGlass with sufficient UV transparency for Cherenkov light collection, clear separation of Cherenkov and Scintillation light of sufficient intensity, low cost, and characterization of CSGlass in the lab and with test beam R&D prototypes. The most critical items are the formulation optimization and production of CSGlass test samples. Some of the CSGlass R&D is shared with SciGlass and PbWO4 crystals for EM calorimeters.  The approximate timeline for completing the CSGlass R&D is ∼3 years assuming R&D funds are available. CSGlass could be ready for future detector upgrades.

Timeline:

| Year | Task |
| --- | --- |
| 2021 | Production of test samples to demonstrate sufficient UV transparency for Cherenkov light collection |
| 2022 | Composition optimization and initial scale up, establish CSGlass characteristics, e.g. measurements of separation of Cherenkov and Scintillation light of sufficient intensity. |
| 2023 | Final formulation optimization and scale up to full module size; establish CSGlass characteristics |
| 2024 | Prototype and beam tests. Process design verification to scale up. |

## Particle ID in Barrel for High-pT

Recent studies showed that by extending the particle ID capabilities of an EIC detector at mid-rapidities out to momenta of 10 GeV/c or higher would allow to expand the EIC’s physics potential substantially. To-date few solutions exist that fit in the given space. New ideas have to be explored on how to reach high-pT PID. Possible solutions could include novel methods such a Long Shadow Detector (LSD) which is in an early conceptual stage.

Timeline: unknown

## Nanowires

Superconducting Nanowire Single-Photon Detectors (SNSPDs) have become the dominant technology in quantum optics due to their unparalleled timing resolution and quantum efficiency. Attempts to transform these sensors into a novel particle detector for the EIC are underway.

These sensors can operate in magnetic fields greater than 5 T at a high rate with very high efficiencies. They provide a timing resolution of better than 20 ps. First R&D effort aims to produce a small superconducting nanowire pixel array for detecting high energy particles.

This first of its kind detector will have the flexibility to be used in multiple far forward detector systems. It can extend the EIC's scientific reach beyond what is possible with contemporary technology for far-forward detection in large parts due to their radiation hardness that allows for a longer service cycle of detectors operating near the beam and interaction regions. One possible implementation at an EIC would include placing the detector inside the magnet and integrating it with the magnet’s cooling system, eliminating the need for a separate cryogenic system.

The required R&D will require several years of generic R&D.

Timeline:

Design of a first generation of individual pixel of few-pixel array detectors can be completed within 2021, depending on when the institutions lift their current work restrictions. After another 6 months to a year, results gathered during that period can be integrated and a process will be developed to fabricate optimized detectors at relevant scales. Thus, after 2 years, the sensor modules of Roman Pot-like superconducting nanowire particle detectors or Compton polarimeter components can be available for full scale prototype testing assuming the availability of appropriate readout electronics.

## Radiation Hard  SiPM

Most likely, all EIC calorimeter technologies will use SiPMs as photosensors, but it is well known that these devices are subject to radiation damage, particularly neutrons. The development of more radiation hard SiPMs would be of great benefit for calorimetry and (if high stability is reached) also for all PID detectors. Developing radiation hard SiPMs would take several years of R&D and require a substantial investment and collaboration with the manufacturers.

Timeline:  May 2021 test beam (protons) …Cost and Schedule

The following table shows the anticipated costs in US$ per fiscal year from FY22 to FY26. It is subdivided into the section discussed above. This list is all-inclusive. The project will fund the necessary R&D projects choosing a selection that is generic and of priority for all detector concepts under consideration.

| R&D Subsystem | FY22 | FY23 | FY24 | FY25 | FY26 |
| --- | --- | --- | --- | --- | --- |
| Reference Detector – Project (Section 1) |  |  |  |  |  |
| Si-Vertex (barrel & disks) w/o sensors | 340,160 | 680,820 | 680,320 | 567,600 | 0 |
| Service Reduction | 42,840 | 85,680 | 85,680 | 71,400 | 0 |
| TPC | 100,000 | 150,000 | 50,000 | 0 | 0 |
| Low-Mass GEM Tracker | 20,000 | 0 | 0 | 0 | 0 |
| Modular RICH (mRICH) | 140,000 | 140,000 | 140,000 | 0 | 0 |
| Dual Radiator RICH (dRICH) | 260,000 | 200,000 | 77,000 | 0 | 0 |
| High-Performance DIRC | 450,000 | 250,000 | 80,000 | 0 | 0 |
| LGAD | 235,000 | 240,000 | 240,000 | 240,000 | 200,000 |
| ECAL: Sc. Glass & Crystals | 90,000 | 50,000 | 50,000 | 0 | 0 |
| ECAL: W powder SciFi, Pb(W)/Sci Shashlik, Lead Glass | 90,000 | 50,000 | 50,000 | 0 | 0 |
| HCAL (forward) | 100,000 | 200,000 | 100,000 | 0 | 0 |
| HCAL (barrel) | 200,000 | 100,000 | 0 | 0 | 0 |
| Low-Q2 taggers | 100,000 | 50,000 | 0 | 0 | 0 |
| Roman Pots | 40,000 | 40,000 | 20,000 | 0 | 0 |
| ZDC | 100,000 | 50,000 | 0 | 0 | 0 |
| Electronics/ASIC | 275,000 | 900,000 | 760,000 | 500,000 | 0 |
| Total (Reference) | 2,583,000 | 3,186,500 | 2,333,000 | 1,379,000 | 200,000 |
| Generic - Alternatives and Improvements to Reference Detector (Section 2) |  |  |  |  |  |
| Si Sensor Development | 740,000 | 852,500 | 1,189,000 | 639,000 | 0 |
| Central Tracker Cylindrical µRWELL | 130,000 | 50,000 | 0 | 0 | 0 |
| Central Tracker Cylindrical Micromegas4 | 10,000 | 50,000 | 5,000 | 0 | 0 |
| GEM TRD4 | 70,000 | 30,000 | 60,000 | 0 | 0 |
| Time-of-Flight4 | 0 | 45,000 | 90,000 | 90,000 | 0 |
| Photosensors4 | 180,000 | 80,000 | 25,000 | 0 | 0 |
| Total (Generic) | 1,130,000 | 1,107,500 | 1,369,000 | 729,000 | 0 |
| Generic - Future Updates and Improvements (Section 3) |  |  |  |  |  |
| MPGD for RICH/Nano-Diamond Materials | 0 | 0 | 70,000 | 130,000 | 130,000 |
| CSGlass for Hadronic Calorimetry | 0 | 30,000 | 130,000 | 200,000 | 180,000 |
| Particle ID in Barrel for High-pT (LSD) | 0 | 50,000 | 200,000 | 350,000 | 250,000 |
| SC Nanowires | 0 | 0 | 90,000 | 95,000 | 100,000 |
| Radiation-Hard SiPM | 0 | 0 | 200,000 | 400,000 | 400,000 |
| Total (Generic Future) | 0 | 80,000 | 690,000 | 1,175,000 | 1,060,000 |

# Appendix

The advisory committee of the Generic EIC Detector R&D Program met over the days of March 24-26, 2021, to review the progress of the various R&D programs presently being funded by the EIC program. When possible, the meetings (both presentations and executive sessions) were attended by members of the EIC Detector Advisory Committee (DAC). The DAC was primarily charged with providing advice about the division of the R&D effort between project R&D, to address any risk in completion of the planned EIC (Reference) detector, and generic R&D, to address developing new technologies that could substantially increase the range and precision of the proposed program of EIC physics measurements if developed in time.

On June 29, 2021 the DAC submitted their report. The table below summarizes their recommendations:

| R&D Topic | Project R&D | Generic R&D |
| --- | --- | --- |
| Si tracker based on 160nm ALPIDE | X |  |
| Si tracker based on 65 nm MAPS/DMAPS |  | X |
| Central TPC: hybrid GEM/Micromegas | X |  |
| Central TPC: μRWELL |  | X |
| Forward/Backward GEM Tracker | X |  |
| Cylindrical Micromegas/Silicon hybrid tracker |  | X |
| Modular RICH (mRICH) | X |  |
| Dual Radiator RICH (dRICH) | X |  |
| High performance DIRC (hpDIRC) | X |  |
| TOF/tracker based on LGAD |  | X |
| TOF/tracker based on LAPPD |  | X |
| GEM-TRD/tracker |  | X |
| EMCal W/SciFi | X |  |
| EMCal Shashlik | X |  |
| Scintillating Glass | X |  |
| Hadron Calorimeter (hCal) | X |  |
| Cerenkov/Scintillating Glass |  | X |
| Far-Forward Detectors (Roman Pots) | X |  |
| Zero-Degree Calorimeter (ZDC) | X |  |
| Low-Q2 Tagger | X |  |
| Compton Polarimeter | X (accel?) |  |
| MCP-PMT/LAPPD |  | X |
| Radiation Hard SiPMs | X |  |
| MPGD-based Photon Detectors |  | X |
| Superconducting Nanowires |  | X |
| Front-end Electronics | X |  |
| Streaming DAQ/Filtering | X |  |
| Background Simulation | X |  |

The provided recommendations are fully aligned with the plan outlined in this document with two exceptions: (i) the current R&D plan does not include any R&D on background simulations since these efforts are now included in the IR design efforts funded by the project, and (ii) the current plan does not consider R&D on radiation hard SiPMs as an urgent item to be supported at this time since efforts in this direction are already under way in Europe that can be leveraged.

# Update April 2025

The R&D plan detailed above was released in August 2021. The release was followed by a call to existing R&D groups and wider community to form groups around selected technologies and submit proposals. In October then the initial decision on the FY22 program were made and the various projects were informed. Each project has a unique identifier of the form eRDNNN, where NNN is a three-digit number. Unfortunately, progress that year was affected by an over 6 month long continuing resolution and milestones had to be adjusted. For the FY23 program the ePIC collaboration was now fully formed, but not all subdetector technologies have been chosen. FY24 saw some changes in the ePIC baseline subsystems and the R&D planning was adjusted accordingly.

During FY23 and FY24, there was significant communication with the community to clarify the distinction between R&D activities aimed at addressing intrinsic detector technology risks and those focused on design maturity. Through this process, we separated earlier milestones, categorizing those more aligned with Project Engineering and Design (PED). Now in FY25 we see the requests for funding for R&D dwindling as expected. Many projects have concluded, and most technical risks (before CD-2 baselining) are mitigated. All of the R&D milestones are projected to be completed in FY25 as further detailed below. Some sensors/ASIC related engineering efforts are to support near-final or pre-production engineering runs and will be supported by PED.

The following tables show the evolution of the R&D program from FY22 to FY25.

| ID | R&D Subject | Funding |
| --- | --- | --- |
| eRD101 | mRICH | $60,000 |
| eRD102 | dRICH | $210,000 |
| eRD103 | hpDIRC | $155,000 |
| eRD104 | Service reduction (Si) | $47,000 |
| eRD105 | SciGlass | $90,000 |
| eRD106 | Forward EMCAL | *** |
| eRD107 | Forward HCAL | *** |
| eRD108 | Cylindrical MPGD | $86,000 |
|  | Planar MPGD | $27,000 |
| eRD109 | ASICs | *** |
| eRD110 | Photosensors | $185,000 |
| eRD111 | Si-Tracker (no sensors) | $240,800 |
| eRD112 | AC-LGAD (incl. ASIC) | $200,000 |
| eRD113 | MAPS/ITS3 | *** |
|  |  | $1,300,800 |

Table 1: FY22 R&D Program. Items marked with *** were postponed until technical decisions for project detector technology baseline (ePIC) was finalized and/or funding limitations

| ID | R&D Subject | Funding |
| --- | --- | --- |
| eRD101 | mRICH | $60,000 |
| eRD102 | dRICH | $190,000 |
| eRD103 | hpDIRC | $230,000 |
| eRD104 | Service reduction (Si) | $87,300 |
| eRD105 | SciGlass | $60,000 |
| eRD106 | Forward EMCAL | $99,100 |
| Table 2: FY23 Funding. eRD107 | Forward HCAL | $94,800 |
| eRD108 | MPGDs | $93,905 |
| eRD109 | ASICs | $569,700 |
| eRD110 | Photosensors | $203,000 |
| eRD111 | Si-Tracker (no sensors) | $86,000 |
| eRD112 | AC-LGAD (incl. ASIC) | $250,000 |
| eRD113 | MAPS/ITS3 | $976,000 |
|  |  | $2,999,805 |

Table 2: FY23 R&D program and funding

For FY24 ePIC, R&D had to adapt to cope with the ePIC technology baseline changes. The mRICH (eRD101) was replaced by a pfRICH and the barrel EMCal technology changed from SciGlass towers to an Imaging Calorimeter interleaving layers of Astropix sensors in PbSciFi Matrix

| ID | R&D Subject | Funding | Comments |
| --- | --- | --- | --- |
| eRD101 | mRICH |  | not baseline |
| eRD102 | dRICH | $140,000 |  |
| eRD103 | hpDIRC | $95,000 |  |
| eRD104 | Service reduction (Si) | $102,500 |  |
| eRD105 | SciGlass |  | not baseline |
| eRD106 | Forward EMCAL |  | no funds requested |
| eRD107 | Forward HCAL | $73,600 |  |
| eRD108 | MPGDs | $35,000 |  |
| eRD109 | ASICs | $817,400 |  |
| eRD110 | Photosensors | $150,000 |  |
| eRD111 | Si-Tracker (no sensors) | $110,000 |  |
| eRD112 | AC-LGAD (incl. ASIC) | $286,000 |  |
| eRD113 | MAPS/ITS3 | $1,305,000 |  |
| eRD114 | pfRICH |  | PED only |
| eRD115 | Imaging Cal | $93,400 |  |
|  |  | $3,207,900 |  |

Table 3: FY24 R&D program and funding

| ID | R&D Subject | Funding | Comments |
| --- | --- | --- | --- |
| eRD102 | dRICH | PED | Transitioned to PED: realization and test of the real-scale engineering test article that should incorporate all the ongoing developments |
| eRD108 | MPGDs | $10,000 R&D | R&D: Funds are to ship the cylindrical muRWELL prototype detector that is assembled at Florida Tech to JLAB, conduct a test in an electron beam at JLAB, and then ship it back to Florida Tech. |
| eRD109 | ASICs | $60,000 R&D | $40K for ALCOR funded as R&D: characterization and test of the new functionalities of ALCOR v3 with the socket test board; $20K for readout funded as R&D: validation of design choices on the dual RICH readout board R&D prototype, including a full test of the card while irradiated; $368K transitioned to the PED phase for the SALSA3 engineering run to provide an engineering test article. |
| eRD110 | Photosensors | $20K R&D | R&D: Completion of the photosensor ageing studies at INFN. Test stand and technical support at BNL was moved to PED |
| eRD113 | MAPS/ITS3 | PED | BNL and LBNL are both engaged in the development and characterization of sensors and ancillary ASICs using PED funding. MIT is funded as PED for development and optimization of the hardware probe cards and the related firmware for the MOSAIX/ITS3 ER2 sensor readout. ORNL is funded as PED for sensor characterization at the die level as a function of temperature and irradiation. |
|  |  | $90,000 R&D |  |

Table 4: FY25 R&D program and funding

A total of 13 R&D projects were supported in FY23, 11 in FY24, and only 5 remain in FY25. The following table list all R&D projects with their respective milestone(s) and indicates the current status as being completed or the remaining steps to projected completion in FY25. For reference see 2024 Detector R&D Day.

| Project ID | R&D Subject | Milestone | Status | Comment |
| --- | --- | --- | --- | --- |
| eRD101 | mRICH | Validate production readiness of a Ring-Imaging Cherenkov Detector as matched with photosensors and readout electronics on the electron-side end cap of the EIC detector, including validation by prototypes that the EIC requirements can be met | Obsolete | Not in baseline (FY23) |
| eRD102 | dRICH | Validate production readiness of a dual Ring-Imaging Cherenkov Detector as matched with photosensors, readout electronics, and integrated cooling on the ion-side end cap of the EIC detector, including validation by prototypes that the EIC requirements can be met | Projected to be completed FY25. Several small prototypes have been made to develop and validate components. Quality control of individual components has been validated. Preliminary definition of the technical specifications of all the dRICH components has been completed. Thermal studies benchmarked with prototypes. A real-scale prototype has been completed with readout and optical components. Validation of dRICH production readiness with the real-scale prototype and realistic component demonstrators projected to be completed July 2025. | Completion of integrated cooling (and thermal treatments) is last step but beyond the R&D phase. |
| eRD103 | hpDIRC | Validate production readiness of a new high-performance DIRC detector by DIRC bar characterization, and vertical-slice prototype cosmic and beam tests including a realistic readout box. | Completed Milestone reached and hpDIRC R&D concluded. Preparation for production readiness of hpDIRC: key components tested on dedicated optical benches; integrated vertical-slice prototype ready for tests within the Cosmic Ray Telescope (CRT) facility. Lens design and performance validated through systematic test bench measurements. First legacy DIRC radiator bars successfully separated and undergoing detailed characterization. | Ready for finalizing design of hpDIRC |
| eRD104 | Service reduction | Develop powering scheme based on serial powering to mitigate service space needs of DC-DC scheme. | Completed. A serial powering scheme has been developed that will make it possible to power a group of up to four EIC Large Area Sensors with a single current loop. This scheme relies on the development of an Ancillary ASIC, separate from the EIC Large Area Sensor. Specifications have been developed, a technology choice has been made, and an initial MPW submission with test-blocks has been made. Further work will be carried as PED. |  |
|  |  | Evaluate radiation tolerant FPGAs and high-speed fiber optic transmission option with beam test. | Completed. The readout and slow control scheme has been developed in conjunction with the serial powering scheme. The core components for the baseline scheme, beyond the Ancillary ASIC, are the VTRx+ optical link module developed for the higher (than for ePIC) radiation environments at the LHC and lpGBT (low power Gigabit Transceiver). Test setups for this scheme and for the originally envisioned alternative scheme based around commercial components are in place at ORNL. Bench tests have been performed. A test board for high-speed data transmission is available at ORNL. |  |
| eRD105 | SciGlass | Scintillating glass scaled up and characterized to >15 X0. Prototype tested in electron beam. | Obsolete | Not in baseline (FY23) |
| eRD106 | Forward EMCAL | Validate production readiness of the EM calorimeter by optimizing uniformity and efficiency of light collection with SiPM readout, integration of final readout, and common beam test with prototype. | Completed |  |
| eRD107 | Forward HCAL | Validate production readiness of the hadron calorimeter by machining and characterization of tiles, prototype module production, integration of final readout, and common beam test with EM calorimeter prototype. | Completed (beyond documenting) Completed beyond work on reports and publication. Evaluation of tolerance compliance for tile production using machining and injection molding completed. First beam test with integration of HGCROC read-out for a full LFHCAL module was completed August 2024, reading every single SiPM (Silicon Photomultiplier). Analysis is ongoing. Test beam analysis from 2023 and 2024 are on-going, aiming at paper combined paper for late summer 2025. | Test beam was no available. However, common beam tests with EMCal in front was treated as optional |
| eRD108 | MPGDs | Complete beam test validation of cylindrical 𝜇RWELL with prototype. | Complete with small-size prototypes. | As part of PED effort, a full-scale engineering test article is developed to verify mechanical stability which is the largest operational issue. |
|  |  | Complete validation of use of light-weight cylindrical Micromegas with 2D readout patterns | Final validation by end of CY2025. The new upgrade for ePIC from the earlier used cylindrical technology (CLAS12) is 2D readout. A 2023 beam test has been completed |  |
| eRD109 | ASICs | Develop a streaming readout solution for the EIC calorimeters with SiPMs and the associated PCBs and cabling infrastructure. Decide between options using COTS devices or directly starting from existing ASICs. | Completed. Discrete implementation with COTS devices was selected for forward and backward EM calorimeters with specifications completed in 2024, beam tests at DESY in 2025; engineering article planned as supported by PED in FY26 and production in FY27. The CALOROC ASIC is under development by OMEGA/IN2P3 group as in-kind, benefitting from evaluation with the H2GCROC ASIC, and selected for the LFHCAL. Tests were performed at CERN in May 2024. The CALOROC is compatible with streaming readout and further development is supported by PED: CALOROC1A and 1B in FY24 – FY25, and CALOROC2 (final version) in FY26. The production run for the CALOROC is foreseen for FY27. | Solutions developed; ASICs (pre-production) engineering runs are PED |
|  |  | Develop a streaming readout solution specific to the Ring-Imaging Cherenkov (RICH) particle identification detectors with SiPMs. This may require a modification of an existing ASIC design. | Completed. The ALCOR ASIC was selected. The ALCOR development is done as in-kind by INFN. ALCORv2 32-pixel 8 x 4 was completed in FY24. ALCORv3 64-pixel 8 x 8 has its engineering run in FY25. Further development would be supported as PED activity as it would be for refinements: ALCORv4 ready for production in FY26. |  |
|  |  | Develop a streaming readout solution specific to particle identification detectors with LAPPDs and/or MCPMTs. This requires a novel ASIC in 130 nm CMOS technology that meets the requirements set by EIC providing a precise time measurement with a TDC combined with an Analog Digital Converter (ADC) for the amplitude measurement. | Completed. A variant of the FCFD ASIC was selected, benefitting from the development of the FCFD (65 nm) for the AC-LGAD strip readout. Further work on FCFD engineering runs will be supported by PED in FY25 and FY26; production of the FCFD variant for the pfRICH and the hpDIRC is planned for FY27. |  |
|  |  | Develop a low-mass streaming readout solution specific to the barrel-region AC-LGAD at an EIC Detector, including design of the power delivery and readout service system. This includes a decision between redesign of existing ASICs or use of third-party ASIC solutions | Completed. The EICROC ASIC has been selected for AC-LGAD pixel readout. The EICROC is based on the previously developed HGCROC ASIC and will conform to streaming readout requirements. The EICROC is done as in-kind by OMEGA/IN2P3. EICROC0 exist with 4 x 4 channels, power reduction is implemented in EICROC0A and 0B. Design of EICROC1A (8 x 32) and EICROC1 (32x32) are complete and ready for submission in FY25. The final version, EICROC2 (32 x 32) will be completed in FY26 with production in FY27. The FCFD has been selected for barrel AC-LGAD strip readout. FCFDv1 has been completed in FY24 with added interfaces in FY24/FY25 (FCFDv1.1) and with synergies with FNAL/DUNE. Work towards FCFDv2, the final version, is supported under PED. The FCFDv2 engineering run will occur in FY26. FCFD is expected to be ready for production in FY27. |  |
|  |  | Develop a streaming readout solution specific to the MPGD-based tracking detectors. This requires a modification based on previous ASIC designs to include compatibility with streaming readout, a new CMOS technology, and 64-channels. | Completed. Much work has been done as in-kind by CEA/USP. A 4-channel SALSA was submitted April 2024 and confirmed viability. The SALSA2 and final SALSA3 implementations in 65 nm technology will be supported as PED in FY25 and FY26, to support 64-channels. Production is anticipated in FY27. |  |
| eRD110 | Photosensors | Ensure applicability of SiPM readout for RICH detectors. Validation of operation in the single photon regime, and studies of irradiation and in-situ annealing to ensure operation in a radiation damaged and annealed mode is possible. | Completed. Some continuing studies driven by INFN. | MCP-PMT tests (HRPPD fallback) |
|  |  | Establish production readiness of a LAPPD/HRPPD-based photon-sensor readout for a Ring-Imaging Cherenkov Detector on the electron-side endcap of ePIC, including validation by prototype beam tests. | Obsolete. Production readiness of HRPPDs is captured as PED effort. Engineering test article of proximity-focusing Ring-Imaging Cherenkov Detector ongoing. Performance evaluation of the first HRPPD engineering test articles is ongoing, including ageing and magnetic field studies at JLab, BNL and INFN-Trieste. Results are promising and show the HRPPD meet EIC detector specifications. A design iteration leading to a small batch second HRPPD engineering test article run is planned for 2025. |  |
| eRD111 | Si-Tracker (no sensors) | Develop barrel and disc cooling options, allowing cooling choice. | Completed (beyond documenting). ALICE ITS3 has demonstrated that air cooling is feasible for their array of vertex layers. SVT will make use of the ITS3 sensor for its Inner Barrel. Air cooling by forced convection internal to the support structures of the SVT Outer Barrel and Disks has been investigated for anticipated power density profiles associated with the EIC Large Area Sensor currently under development. This has been done with thermomechanical mockups and, in select cases, cross-validated with ANSY simulations. Air cooling is considered a viable option for SVT operations near ambient temperatures, provided that the power dissipation in the EIC LAS sensor can indeed be reduced as projected compared to the MOSAIX periphery and that power dissipation in the Ancillary ASIC will be well below that of the EIC LAS itself. Liquid cooling has material and service load implications and trade-offs in its use in strategic places are being investigated. |  |
|  |  | Develop techniques for stitching of sensors | Completed. Tiling studies on the staves of the SVT Outer Barrel and the Disks, combined with current foundry insights, have shown that single segments with 5 or 6 stitched Repeated Sensor Units will result in an EIC LAS that meets SVT acceptance and other requirements. For reference, the SVT Inner Barrel will make use of wafer-scale MOSAIX sensors that contain three to five segments each with 12 stitched Repeated Sensor Units. With this specification for the EIC LAS, the SVT will have approximately 4,000 EIC LAS. | Developing EIC LAS through modification of MOSAIX requires access to the MOSAIX design database (technology transfer) and is subject to a CERN-EIC agreement. This currently hampers progress. |
| eRD112 | AC-LGAD (incl. ASIC) | Validate production readiness of the AC-LGAD detectors for ToF in central detector and the far-forward Roman Pot beam line detectors. This includes sensor prototypes, ASIC demonstrator, and full-scale parts tested and finalized. | Small sensor productions look promising. Have purchased 3rd version HPK sensors with FY24 funds and collaborated on CERN purchase of FBK sensors for comparison, testing ongoing in FY25. ASIC demonstrator can only be beyond CD-2 so this will be Project Engineering and Design. First tests of full-size HPK strips carried out; order placed but still awaiting first full-size pixels from HPK. BNL sensors are tested along HPK. Design and fabrication of new AC-LGAD sensors to improve performance and obtain compatibility with EICROC1 (when available) is shifted to PED. | Sensors and parts tested; sensor (pre-production) ERs are PED |
| eRD113 | MAPS/ITS3 | EIC vertex sensor quantification finalized | Projected to be completed this FY25. Tape-out of ER2/MOSAIX at CERN is currently anticipated to be in May or June 2025. Samples are then expected by year-end and initial validation a few months after. Work beyond ER2 will be carried as PED, since this is towards production-readiness sensors. | Anc. ASIC R&D and sensor characterization; (pre-production) ERs are PED |
| eRD114 | pfRICH | N/A | N/A | Moved to PED from beginning. Critical item of pfRICH is the HRPPD sensor that was covered by eRD110. |
| eRD115 | Imaging Cal | Characterize the integrated AstroPix and SciFi/Pb system with a mixed e/π beam and MIPs, benchmarking the response to charged pions, benchmarking the electron-pion separation capability, and testing the new generation SiPMs. | Project completed in FY25. The GlueX Baby BCal, equipped with a single-layer AstroPix sensor, was tested and benchmarked at the Fermilab Test Beam Facility (FTBF) in FY24 using a mixed electron/pion beam and MIPs, despite very limited beam time availability. The data have been analyzed, benchmarking the response to electrons and pions, as well as the e/π separation performance in the SciFi/Pb section of the prototype. AstroPix v3 was extensively tested against design requirements. A proof-of-principle integration was successfully demonstrated both in the beam and on the bench. Thin SciFi/Pb layers with a new SiPM readout were also commissioned in a bench setup. | No beam tests are expected at FTBF in FY25. Larger-scale readout integration studies are beyond the scope of the R&D effort and are instead part of the ongoing PED plans. |

The following graph indicates the actual R&D funding compared to the one envisioned in 2021 (see Chapter 4). We note that as compared to the initial projected EIC detector R&D planning, less integrated funding was used to accomplish the main R&D goals. This is due to two main factors. The first factor was that the EIC detector R&D was started at lower than foreseen funding. This was partly due to giving overall EIC Project priority to accelerator-related R&D, partly to align with the ongoing EIC detector proposal process that was spanning 2021 and early 2022 and ensure R&D was aligned with technical baseline decisions for the EIC detector/ePIC. The second factor was induced by overall EIC Other Project Cost funding limitations. This led to a general decision that early Silicon-chip related engineering runs (work towards MAPS/ITS3 and AC-LGAD sensors, and towards various ASICs for electronics readout) to prove the planned chips were compatible with the foreseen EIC requirements and streaming readout are treated as R&D, whereas later engineering runs folding in design modifications, increases in channel density, and final functionality pre-production tests are treated as engineering test articles funded as Project Engineering and Design.

The only official copy of this document is the one online in the SharePoint Document Center. Before using a printed copy, 
verify that it is current by checking the printed document’s Version History log with that of the online version.

| Electron Ion Collider, Brookhaven National Laboratory |  |  |  |
| --- | --- | --- | --- |
| Doc No. [Insert here] | Author: [Insert here] | Effective Date: [Insert here] | Review Frequency: [Insert here] |
| Policy: [Insert Title] |  |  | Version: [Insert here] |

The only official copy of this document is the one online in the SharePoint Document Center. Before using a printed copy, 
verify that it is current by checking the printed document’s Version History log with that of the online version.

| Electron Ion Collider, Brookhaven National Laboratory |  |  |  |
| --- | --- | --- | --- |
| Doc No. [Insert here] | Author: [Insert here] | Effective Date: [Insert here] | Review Frequency: [Insert here] |
| Policy: [Insert Title] |  |  | Version: [Insert here] |

Given the status of planning in the proto-collaborations a TPC is increasingly unlikely to be used which would eliminate it from the R&D program.

200k is for MCP-PMTs to be used in prototypes. They are likely to be used in final detector.

Referring beam line detector (Roman Pots) will/can come in late.

Given the status of the discussions within the proto-collaborations this project could become generic and project funded.

https://wiki.bnl.gov/conferences/index.php/EIC_R%25D