EIC-Project meets ePIC DSCs: fECal

March 4th, 2025

- Josh presentation:

Units in tables are in inches, the tolerances need to be re-evaluated, the tolerances between the ¾ of a mm

sPHENIX axhieved a +/- ¼ mm on the block  80% yield

Oleg’s goal is 1/4000 of an inch on block tolerances

Tolerance on the mold is 1/2000

What was the precission on the mold for sPHENIX

Invite Eric to tell us about lessons learnt on tolerances

need to do a pre-production of the blocks to understand the tolerances

Plan use some of the fudan blocks and machine them to the desired tolerances and use them to understand the achivable tolerances of all the other blocks

In parallel build some blocks to understand the tolerances of the production process.

Note: Simulation studies showed that for EEEMcal we found 0.5mm no effect. So, I guess for fECal (poorer resolution) 1mm should be ok

Action item:

Adapt 1mm space between bocks  Akio simulates this and only if there is a large impact we rediscuss this spacing

Josh re-does all his tables

Josh will be doing the drawings for the gluing fixtures.

Design Requirement:

The boards cannot be split as that would limit the space to fit all components on one board to fit in the minimum space in z

The south side boards are rotate 180 degree to the north side boards

Document the numbers on sizes and so on

Cooling:

Need requirements for the cooling system need to be filled in the sent out excel-form

EIC-Project meets ePIC DSCs: fECal

November 15, 2024

- SiPM board

Model is well defined (by Oleg, Gerard et al), and it should be implemented in CAD

-> Josh will start working on it

-> Josh requests a follow-up meeting next week (include Oleg, Gerard, Dan, Rahul)

- Glue

Used to flue SiPM board to LG, and then connectors to SiPM boards.

Is glue strong enough? – Oleg: yes, too strong

If SiPM board fail, can it be dis-assembled without breaking anything (e.g. LG)?

Any alternatives to glue? – Need an optical contact to LG

-> Gerard et al will study it in the next a few weeks

- Cooling

Cu tubing. Any requirements for cooling water? – Looks no.

Now much mass the Cu tubing adds to the system? – Needs to be implemented and verified in CAD.

Cooling system as a whole (number of chillers, etc) need to be further developed and optimized without compromising the level of required cooling of different elements.

Any specs for cooling system performance (e.g. T difference between different parts/elements)?

- Module stacking

Is the designed 1/4mm between modules enough tolerance for module stacking? Is it achievable?

-> Dan will check it by the mid of Dec.

- Cooling&Cabling

They are inside the box (8cm). Need to double check the envelope is consistent with CAD implementation (now 19+8cm); Oleg: the module itself is 17cm of W/SciFi + a few mm backplate (so, 19cm looks safe enough)

-> Dan will double-check it

All cables are suggested to get out at 3 and 9 o’clock – may be a challenge for fECal design (want more exit points).

-> Let Roland route the cables&pipes, and then call another meeting to discuss these.

Let Roland know if there are any changes relative to design/geometry summarized in epic interface document (link given by Roland)

Dan: with cable penetration, is the system light closed? – Oleg: we have a simple solution

Elke: Everything should get in CAD model!