CERN test beam list of items

- 4 trackers + 4 DUTs

- HV channels:

  - 4 Trackers: 4

  - DUTs:  16

  - Ideally two double sided with one reading out one detector at a time.

    - 2 set up with one small detector with pad readout  (8 APVS) and other will be large chamber + 2 small single thin gap chamber

    - JLAB provide 12 channels.

- HV cables:

  - 4 trackers: 4

  - 2 DUTs: 16

- APVs:

  - 4 Trackers:  8 (4 pairs Master -Slave)

  - DUTs

    - Small thin gap pad readout – 8 APVs

    - Large double side  - 8 APVs

    - Probably can get 5 Masters + 5 slaves from Vanderbilt APVs

- HDMI cables (longest one available) for

    - 8 NEEDED and all   of the HDMI cables can be taken from Hall D TRD project and some spare.

- Flat ribbon cables for APV Master slave (longest available)

  - Based on configuration of Master slave from previous bullets 8 ribbons

- Scintillator+ PMT  for trigger : 2

- SRS system : 1 (One from Hall D) + may be 1 from UVA additional

- DAQ computer : 1

- Ethernet cable : 2-3

- DUT support structure using  80/20  frame: 1 (40 cm x 40 cm x 40 cm)

- Tracker support structure using 80/20 frame : 2 (40 cm x 40 cm x 20 cm)

- Gas tubing (?)

- Some hardware like screw driver etc

- Network switch

- External hard disc ( 2-4 TB)

- Lemo cables

- Twisted cables  (Xinzhan)

- Ask CERN to provide 2 NIM bin slots .