## OB QC steps

This is a first stab at a set of QC instructions. They are incomplete, as the design for the OB staves and their supports are still evolving and will need to be updated accordingly.

### Component Acceptance

- Electrical test of FPC for connectivity and isolation. At Oxford we would use the existing tape testing robot from the ATLAS assembly.

  - FPC test probably will include FIB.

- We assume that modules (incl. sensors and AncASICs) have been fully tested, following procedures outlined in their specific QC documents.

- Dimensionally relevant parts (OB: cross-ribs, stave end pieces) will be batch-surveyed.

### Mechanics Assembly

- Weights will be tracked during all assembly steps, to keep track of glue weights.

- To be confirmed at a later time: Verify electrical connection or isolation of the completed support structures (as needed).

- Tap test of completed assembly.

- 3pt bend test of completed assembly.

- Survey of key dimensions using CMM or equivalent survey technology.

  - Includes survey of contact areas with module.

  - Includes mount points.

- Possible: Optical survey of FPC bond pads (for connection to module FPC).

- Possible: envelope check of structure before module loading.

- Functionality check of FPC (possibly using the tape robot?).

### Module Gluing & Electrical Integration

- Verify integrity of air channel: flow air through channel with flow meters at input and output and compare readings.

- Verify module operation following QC tests outlined by module community.

  - During operation monitor temperatures using a thermal imaging camera (for a given air flow).

- Mounting of loaded stave in a mechanical reference setup, envelope check.

- Ideally, no test of module after mounting, only for completed stave.

Test of full stave described in separate document