Meeting Notes:

Endcap Racks:

- Will we need access to the front and back often?

  - Probably, yes. Will likely need ample space to work on both sides of the racks as well. Stacking may make this tricky.

- Can we stack the racks if needed?

  - Yes, as long as there is access and space to work.

- How often can we disconnect the fibers, power and cooling? Is power and cooling needed in the open positions?

  - Would like to limit disconnections as much as possible. Treat the connectors as fragile.

  - We will need likely need separate cooling and power hookups for the racks. The fibers will need clever routing such that it doesn’t need to be disconnected to move.

  - Length of the fibers is not an issue here, we will be around +300ft for the forward side.

  - ME group needs to figure out how to cool and power the racks when they are in the closed position, open position, and when the main detector is rolled out.

- Are 2 racks on the forward side per half and 1 rack on the backward side per half enough?

  - It appears that we will likely need more rack space on the forward side but the EE group will get back to us on this.

North and South Platforms

- John spoke with Pat regarding the clearances needed for racks and their doors, Pat said that 36” were needed for clearance including when the doors are open. Meaning that we need up to 60” (36” + door) of clearance. Can we get away without doors?

  - We don’t need doors, this may actually be better. Need to look into if each crate has a heat exchanger or if its only one heat exchanger per rack.

- Number of Racks?

  - Seems like we will have plenty of racks and wont need the 3rd level half racks, but the EE group needs to do some work on rack assignments.

- Cooling pipe materials?

  - Copper or whatever material up till the racks is fine. The jumpers from the main pipes to the racks can be a nonconductive material.

- Rack Isolation?

  - We need to isolate the racks. We need to find out if the STAR racks were already isolated or not.

- Magnet group racks?

  - The magnet group needs racks on the North platform. Not sure how many but we think 3? They need clean power, likely separate from the electronics power. We need to find space for these. Steve will ask Renuka.